Aplicaţia 1

Circuit pentru afişarea cifrelor în ordinea 1234 pe display-ul plăcii FPGA

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_arith.all;

use ieee.std_logic_unsigned.all;

entity s3_display1234 is

port ( CLKIN : in std_logic;

AN3 : inout std_logic;

AN2 : inout std_logic;

AN1 : inout std_logic;

AN0 : inout std_logic;

LED : out std_logic_vector(6 downto 0));

end s3_display1234;

architecture behavioral of s3_display1234 is

signal CTR : STD_LOGIC_VECTOR(12 downto 0);

begin

Process (CLKIN)

begin

if CLKIN'event and CLKIN = '1' then

if (CTR="0000000000000") then

if (AN0='0') then

AN0 <= '1';

LED <= "0000110";           -- cifra 3

AN1 <= '0';

elsif (AN1='0') then

AN1 <= '1';

LED <= "0010010";             -- cifra 2

AN2 <= '0';

elsif (AN2='0') then

AN2 <= '1';

LED <= "1001111";             -- cifra 1

AN3 <= '0';

elsif (AN3='0') then

AN3 <= '1';

LED <= "1001100";             -- cifra 4

AN0 <= '0';

else

LED <= "1001100";             -- cifra 4

AN0 <= '0';

end if;

end if;

CTR<=CTR+"0000000000001";

if (CTR > "1000000000000") then   -- num. ajunge la 2^13

CTR<="0000000000000";

end if;

end if; -- CLK'event and CLK = '1'

End Process;

End Behavioral;

- Să se editeze în VHDL o entitate de test numită test_display1234 pentru a simula şi verifica funcţionalitatea entităţii s3_display1234.

- Să se sintetizeze entitatea s3_display1234 cu programul Xilinx ISE şi să se implementeze rezultatul în placa FPGA Spartan 3 folosind următoarele asocieri ale semnalelor cu pinii FPGA :

Asocierea semnalelor cu pinii

NET "AN0"  LOC = "D14"  ;

NET "AN1"  LOC = "G14"  ;

NET "AN2"  LOC = "F14"  ;

NET "AN3"  LOC = "E13"  ;

NET "CLKIN"  LOC = "T9"  ; clock 50 MHz

NET "LED<0>"  LOC = "N16"  ;g

NET "LED<1>"  LOC = "F13"  ;f

NET "LED<2>"  LOC = "R16"  ;e

NET "LED<3>"  LOC = "P15"  ;d

NET "LED<4>"  LOC = "N15"  ;c

NET "LED<5>"  LOC = "G13"  ;b

NET "LED<6>"  LOC = "E14"  ;a

Aplicaţia 2 –  Afişare pătrate pe un monitor VGA

Scopul aplicaţiei este de a desena pe un monitor VGA pătrate cu lăţimea de 8 pixeli.

Fig. 1. Asocierea pinilor FPGA la portul VGA

Fig. 2 Codarea culorilor  pe 3 biti

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity vga_squares is

port(clk50_in  : in std_logic;

red_out   : out std_logic;

green_out : out std_logic;

blue_out  : out std_logic;

hs_out    : out std_logic;

vs_out    : out std_logic);

end vga_squares;

architecture Behavioral of vga_squares is

signal clk25              : std_logic;

signal horizontal_counter : std_logic_vector (9 downto 0);

signal vertical_counter   : std_logic_vector (9 downto 0);

begin

-- generate a 25Mhz clock

process (clk50_in)

begin

if clk50_in'event and clk50_in='1' then

if (clk25 = '0') then

clk25 <= '1';

else

clk25 <= '0';

end if;

end if;

end process;

process (clk25)

begin

if clk25'event and clk25 = '1' then

if (horizontal_counter >= "0010010000" ) -- 144

and (horizontal_counter < "1100010000" ) -- 784

and (vertical_counter >= "0000100111" ) -- 39

and (vertical_counter < "1000000111" ) -- 519

then

red_out <= horizontal_counter(3)

and vertical_counter(3);

green_out <= horizontal_counter(4)

and vertical_counter(4);

blue_out <= horizontal_counter(5)

and vertical_counter(5);

else

red_out <= '0';

green_out <= '0';

blue_out <= '0';

end if;

if (horizontal_counter > "0000000000" )

and (horizontal_counter < "0001100001" ) -- 96+1

then

hs_out <= '0';

else

hs_out <= '1';

end if;

if (vertical_counter > "0000000000" )

and (vertical_counter < "0000000011" ) -- 2+1

then

vs_out <= '0';

else

vs_out <= '1';

end if;

horizontal_counter <= horizontal_counter+"0000000001";

if (horizontal_counter="1100100000") then

vertical_counter <= vertical_counter+"0000000001";

horizontal_counter <= "0000000000";

end if;

if (vertical_counter="1000001001") then

vertical_counter <= "0000000000";

end if;

end if;

end process;

end Behavioral;